Seismic Workbench

Technical Project Report

A Lightweight Seismic Processing & Interpretation Platform

Rudra Sharma

IIT Roorkee  |  Geophysics, Final Year

June 2026

# Table of Contents

# 1  Project Overview

The Seismic Workbench is a Python package and Streamlit web application covering the full exploration-geophysics workflow from raw SEG-Y data to interpreted fault maps. It was designed as a portfolio project demonstrating real processing competence — the kind that operates on the underlying physics, not just data-analysis notebooks.

## 1.1  Key Design Decisions

- SEG-Y I/O via segyio only. ObsPy was excluded because it targets earthquake seismology (miniSEED, FDSN), not reflection seismology. It adds a heavy dependency for nothing.

- Synthetic ground truth. The pre-stack pipeline is validated against a built-in convolutional forward model with known RMS velocities. Semblance recovers true v_rms within 5%; NMO flattens events within ±2 samples.

- Correct dataset matching. F3 Netherlands and Penobscot are post-stack volumes — they have no offsets and cannot feed velocity analysis or NMO. Pre-stack work uses synthetics; post-stack attributes and coherence use F3 (Zenodo record 3755060).

- Streamlit front-end. Fastest path for upload / view / process / QC. Interactive picking is handled via an editable data table.

- Scope discipline. ML fault/salt detection, seismic inversion, 3D rendering (PyVista), and a full interpretation workspace are explicitly listed as future work. Each is a separate multi-month project.

# 2  Tech Stack

| Component | Details |
| --- | --- |
| Language | Python 3.10 |
| Core libraries | NumPy 1.26, SciPy 1.13, segyio 1.9+ |
| Visualization | Matplotlib 3.8, Plotly 6.6 |
| Front-end | Streamlit 1.46 |
| Testing | pytest 8.2 — 37 tests, 0 failures |
| Dataset (F3) | 401 inlines × 701 crosslines × 255 samples, dt = 4 ms, amplitude ∈ [−1, 1] |

# 3  Repository Structure

D:\Downloads\seismic-workbench\

seisproc\                     Core library (8 modules)

synthetic.py                Ricker wavelet, convolutional model, CMP gathers, synthetic 3D volumes

segy.py                     SEG-Y read/write (unstructured and 3D geometry-aware)

processing.py               Bandpass filter, per-trace normalisation, AGC

velocity.py                 Semblance spectrum, auto-pick

nmo.py                      NMO correction with stretch mute, fold-normalised stack

attributes.py               Envelope, instantaneous phase/frequency, windowed RMS

coherence.py                C2 semblance coherence, fault likelihood helper

volume.py                   3D Volume class, SEG-Y and .npy I/O, inline/crossline/time-slice accessors

app\streamlit_app.py          Two-mode Streamlit application

tests\                        37 pytest tests (0 failures)

scripts\download_f3.py        Zenodo F3 benchmark fetcher (~1.2 GB)

data\f3\                      F3 Netherlands volume (546.9 MB train seismic, on disk)

ROADMAP.md                    Locked design decisions and milestone plan

# 4  Milestone 1: Pre-Stack Pipeline

Goal: demonstrate the core exploration-geophysics competence — load, view, filter, analyse velocities, apply NMO, and stack CMP gathers. The pipeline is validated against a synthetic forward model so correctness is provable, not merely plausible.

## 4.1  Synthetic Forward Model

- Ricker wavelet at configurable peak frequency (default 25 Hz).

- Reflectivity spike series convolved with wavelet (convolutional model).

- CMP gather generator: hyperbolic moveout t(x) = √(t₀² + x²/v_rms²), configurable layers (t₀, v_rms, amplitude), offsets, additive white noise.

- Default 5-layer earth model with RMS velocities 1600–2700 m/s.

## 4.2  SEG-Y I/O

- Reads any unstructured (trace-sequential) SEG-Y: traces, binary/trace headers, text header, offset attribute.

- Round-trip write/read preserves amplitude data to float32 precision.

- Metadata summary includes: n_traces, n_samples, dt, offset range, CDP range, format code.

## 4.3  Signal Processing

- Zero-phase Butterworth bandpass (sosfiltfilt) with configurable corner frequencies and filter order.

- Per-trace normalisation: RMS or peak-absolute.

- AGC: sliding-window RMS gain (uniform_filter1d), configurable gate length).

## 4.4  Velocity Analysis

- Semblance spectrum S(t₀, v) computed over a user-defined velocity grid with a configurable smoothing window.

- Auto-pick: scipy.signal.find_peaks on the best-velocity trace, with minimum semblance threshold and minimum event separation.

- Results loaded into an editable Streamlit data table — rows can be added, edited, or deleted before applying NMO.

## 4.5  NMO Correction and Stack

- Per-trace interpolation onto the NMO-corrected time axis using numpy.interp.

- Stretch mute: samples where (t(x) − t₀)/t₀ exceeds the configured threshold are zeroed.

- Fold-normalised stack: mean over live (non-muted) samples per time step.

## 4.6  Validation Results (M1 Exit Criteria)

- Semblance peaks recover true v_rms within 5% for all 5 synthetic layers.

- NMO correction flattens every reflector to within ±2 samples (4 ms) of t₀.

- Stacking 30 noisy traces improves SNR by more than 2× over a single trace. The theoretical optimum is √30 ≈ 5.5×; the test asserts a conservative 2× to be noise-seed independent.

# 5  Milestone 2: Post-Stack Attributes

## 5.1  3D Volume Support

- Volume class: (n_ilines, n_xlines, n_samples) float32 array with inline, crossline, and time-slice accessors.

- Geometry-aware SEG-Y read via segyio.tools.cube; round-trip write preserves data to float32 precision.

- .npy loader (from_npy) for the F3 benchmark format.

## 5.2  Complex-Trace Attributes (Hilbert Transform)

All attributes are computed via scipy.signal.hilbert along the time axis and work unchanged on traces, 2D sections, and full 3D volumes.

- Envelope (reflection strength): |hilbert(trace)| — highlights bright spots and impedance contrasts.

- Instantaneous phase: angle(hilbert(trace)) — reveals lateral continuity regardless of amplitude.

- Instantaneous frequency: d/dt(unwrapped phase) / 2π, with short smoothing window to suppress division-near-zero spikes — low-frequency shadow identifies gas-bearing zones.

- Windowed RMS amplitude: sqrt(uniform_filter(trace², window)) — a standard reservoir proxy.

## 5.3  Synthetic 3D Ground Truth

A 40 × 50 × 251 sample volume with a structural dome (Gaussian time pull-up) and a bright-spot amplitude anomaly (2.5× boost) on the middle reflector. Validation confirms that envelope recovers AM modulation within 5% and the bright spot reads more than 2× background amplitude in the envelope volume.

## 5.4  F3 Netherlands Dataset

- Source: Alaudah et al. (2019) facies classification benchmark, Zenodo record 3755060.

- Volume dimensions: 401 inlines × 701 crosslines × 255 samples, dt = 4 ms, amplitude normalised to [−1, 1], float64.

- Files on disk: train_seismic.npy (546.9 MB), test1_seismic.npy (272.8 MB), test2_seismic.npy (233.8 MB) plus corresponding label volumes.

# 6  Milestone 3A: Coherence Fault Highlighting

## 6.1  Algorithm — Marfurt-Style C2 Semblance Coherence

For N spatially neighbouring traces aⱼ(t) in an (inline, crossline) cell, the semblance coherence is defined as:

C(t) = Σ_w [ mean(aⱼ) ]²  /  Σ_w [ mean(aⱼ²) ]

where Σ_w denotes vertical window summation. Both numerator and denominator reduce to uniform-filter operations, making the entire volume computation three sequential scipy.ndimage.uniform_filter calls. Coherence C → 1 for laterally continuous reflectors and drops sharply across faults, channels, and stratigraphic edges. Fault likelihood is defined as (1 − C) rescaled to [0, 1].

Parameters:

- window: vertical analysis gate in seconds (default 20 ms).

- cell: spatial box size in traces (default 3 × 3 in 3D, 3 in 2D).

## 6.2  Synthetic Faulted Volume (Ground Truth)

- 40 × 60 × 251 samples, 4 ms dt, 4 flat reflectors at 0.25, 0.45, 0.65, and 0.85 s.

- Vertical fault at crossline 30: the downthrown side is shifted deeper by 24 ms (6 samples).

- Validation: coherence profile argmin locates the fault within ±1 crossline; fault coherence is less than 70% of the away-from-fault background; the low-coherence lineament is continuous along every interior inline on the event time slice.

## 6.3  Application Integration

The post-stack mode attribute selector now includes “Coherence (semblance)” alongside the Hilbert attributes. When selected, two display modes are available:

- Standalone coherence display: grey-scale with white = coherent (C ≈ 1), dark = discontinuous (C ≪ 1).

- Amplitude co-rendering with fault overlay: amplitude section in grey with low-coherence regions (below a user-adjustable threshold, default 0.5) rendered as a red transparency layer, following the same principle as Petrel’s co-rendering workflow.

# 7  Test Suite

Total: 37 tests, 0 failures. Run with pytest tests/.

| Test module | Tests | What is proved |
| --- | --- | --- |
| test_synthetic.py | 5 | Ricker peak, dominant frequency, gather shape, hyperbolic arrival time, noise reproducibility |
| test_segy.py | 2 | float32 round-trip, metadata summary fields |
| test_processing.py | 4 | Bandpass pass/reject, bad-corner error, RMS/peak normalisation, dead-trace safety, AGC decay balance |
| test_velocity_nmo.py | 5 | Semblance recovers v_rms within 5%, NMO flattens events ±2 samples, stack SNR >2× single trace, stretch mute, velocity interpolation |
| test_attributes.py | 6 | Envelope recovers AM modulation, inst. frequency of tone, phase at Ricker peak, RMS = A/√2, 3D shape, bright spot in envelope |
| test_volume.py | 4 | 3D SEG-Y round-trip, slice indexing, .npy loader, volume summary |
| test_coherence.py | 6 | Range and shape, flat layers coherent, fault at correct crossline ±1, lineament continuity, fault likelihood rescaling, 2D section support |
| test_app.py | 5 | Pre-stack smoke, post-stack smoke, attribute switch, coherence overlay, time-slice coherence overlay |

A key principle throughout: tests validate the physics, not only the plumbing. The velocity analysis test verifies that picked velocities match true model values to within 5%; the NMO test confirms that reflectors are flat in sample-space; the stacking test measures actual SNR improvement. These checks cannot be passed by a correctly-structured but physically wrong implementation.

# 8  How to Run

## 8.1  Install Dependencies

pip install -r requirements.txt

Requirements: numpy≥01.26, scipy≥1.13, segyio≥1.9, matplotlib≥3.8, pandas≥2.0, streamlit≥1.40, pytest≥8.0.

## 8.2  Start the Application

streamlit run app/streamlit_app.py

The sidebar mode toggle switches between Pre-stack (gathers) and Post-stack (volume). The pre-stack mode accepts a synthetic generate or a SEG-Y upload; the post-stack mode accepts a synthetic volume, a 3D SEG-Y, or the F3 .npy files.

## 8.3  Download Real F3 Data (Optional, ~1.2 GB)

python scripts/download_f3.py data/f3

Then in the app: Post-stack mode → F3 .npy volume → data/f3/data/train/train_seismic.npy.

## 8.4  Run Tests

pytest tests/

# 9  Future Work

The following features were scoped out deliberately. Each represents at minimum a multi-week separate project and is listed here as future work, not as incomplete deliverable.

- Seismic inversion: convert amplitude sections to acoustic impedance.

- ML-based fault and salt body detection (U-Net, CNN — each requires separate training data and evaluation pipeline).

- Reservoir property prediction from seismic attributes (porosity, lithology, net-to-gross).

- 3D rendering with PyVista.

- Full interpretation workspace with project save/load, multi-horizon management, and annotation export.

- Flask/React front-end rewrite for production deployment.

# 10  References

Alaudah, Y., Michałowicz, P., Alfarraj, M., Alregib, G. (2019). A Machine Learning Benchmark for Facies Classification. Interpretation, 7(3), SE175–SE187.

Dataset: https://zenodo.org/record/3755060

OpendTect F3 Netherlands Open Dataset. dGB Earth Sciences.

Marfurt, K. J., Kirlin, R. L., Farmer, S. L., Bahorich, M. S. (1998). 3-D seismic attributes using a semblance-based coherency algorithm. Geophysics, 63(4), 1150–1165.